Objective-Based Methodology

The system utilizes a 100% offline Architecture. It operates on a Master-Slave configuration over a closed Local Area Network (LAN).

Objective 1: Collect energy predictions from the hybridized model and combine them with real-time environmental data from system sensors

- Real-Time Data Collection (Physical Sensors): An ESP32 microcontroller acts as the perception node. It interfaces with a PZEM-004T module to measure real-time AC  . A high-precision voltage divider monitors the inverter battery’s State of Charge (SoC). Contextual data is gathered using a DHT11 sensor (temperature/humidity) and RCWL-0516 sensors (to calculate directional room occupancy).

- The ESP32 packages this real-time data into a JSON payload and publishes it over a local Wi-Fi network using the MQTT protocol to an Eclipse Mosquitto broker hosted on the Raspberry Pi 5.

- Hybrid Model Prediction: A background Python script on the Raspberry Pi subscribes to the MQTT broker, logging the real-time data into a local SQLite database. Simultaneously, the pre-trained hybridized AI model (TE-GRU + LightGBM) runs an inference cycle locally on the Pi's ARM processor. It analyzes the historical load trends from the database to generate the Predicted Upper Bound Load for the incoming hour.

- Data Fusion: The Python script comparesthe incoming real-time sensor array (Battery SoC, Occupancy, Actual Load) with the hybridized model prediction.

Objective 2: Create a system that classifies system load into three modes: High Demand (mode A), Normal Load (mode B), and Low Load (mode C)

Load Tiering: The load is grouped into three classes based on their power consumption;

- Class C (Low Load / Essential): Low-power devices like LED lights and fans (~200W).

- Class B (Normal Load / Comfort): Medium-power devices like deep freezers (~500W).

- Class A (High Demand / Heavy): Inductive loads like water pumping machines and HVAC units (1,000W - 3,000W).

The Python Optimization Logic: The data from Objective 1 is fed into the python baackend running on the Raspberry Pi. It evaluates the constraints(e.g., Is the Actual Load exceeding the Predicted Upper Bound? Is the Battery SoC below 40%? Is the occupancy zero?).

Mode Classification Output: Based on these constraints, the algorithm determines the required energy state:

- Mode A (Maximum Allocation): All relays (Classes A, B, and C) are authorized to run.

- Mode B (Average Allocation): Class A heavy loads are shed. Only Classes B and C are authorized.

- Mode C (Conservation Allocation): Classes A and B are shed. Only Class C essentials are authorized to prevent inverter failure or total battery depletion.

Objective 3: Implement switching between energy distribution modes based on predictive and real-time data while considering 3-time battery lag to confirm stability

- The 3-Time Battery Lag Algorithm:The system logs the target mode every 5 minutes. Before switching to the target mode, the State of Charge (SoC) of the battery must be checked 3 times at 5-minute intervals. If the SoC is stable across these three consecutive cycles (representing 15 minutes), the system successfully switches to the target mode; if it is not stable, it does not switch.

- Physical Actuation (Direct GPIO Control): Once the 3-Time Lag condition is satisfied, the Raspberry Pi utilizes its internal GPIO pins to send 3.3V digital signals directly to an array of industrial Solid State Relays (SSRs).

- Relay Sizing for Safety: For High Demand (Class A) appliances like the Water Pump and HVAC, 40A SSRs (Fotek SSR-40DA) on aluminum heatsinks are utilized to safely absorb massive inductive inrush currents. For Class B and C loads, 25A SSRs are utilized, ensuring high-margin industrial safety across the distribution board.

Objective 4: Develop a mobile/web flutter dashboard that displays real-time system status and distribution activity

- UI Development: The dashboard was programmed using the Flutter framework (Dart language), designed with a responsive layout to adapt to both mobile screens and laptop browsers.

- Web Compilation and Edge Hosting: The Flutter application was compiled into static, minified web files (HTML, CSS, JS). These files were deployed to an NGINX web server installed locally on the Raspberry Pi 5. The system operates on a closed local area network provided by a standard TP-Link 2.4GHz router.

- Real-Time Data Streaming: To achieve instant data visualization, the Flutter web application utilizes MQTT over WebSockets. It acts as an MQTT Subscriber, directly listening to the Mosquitto broker on the Raspberry Pi. When the ESP32 publishes new sensor data, or the Pyomo algorithm updates the active Mode, the UI updates synchronously in milliseconds without requiring page refreshes or internet connectivity.